TECHNICAL BASELINE RECOVERY & HARDWARE AUDIT REPORT:
DISCRETE OPTICAL BRAILLE CHARACTER RECOGNITION SYSTEM

Project Status Audit, Hardware Extraction, and Research Scoping Baseline
Department of Embedded Technology, SENSE, Vellore Institute of Technology (VIT)

1.  ACTUAL EXISTING PROJECT SUMMARY

The current physical project is a benchtop, breadboard-implemented prototype of an embedded Braille-to-text converter. The system utilizes an Arduino Nano (ATmega328P microcontroller running at 16 MHz with CH340 USB interface and Old Bootloader), six discrete TCRT5000 reflective infrared sensor breakout modules, and a 16x2 character LCD interfaced via a PCF8574 I2C backpack.

The system reads digital binary outputs (DO) from the six TCRT5000 onboard LM393 comparators, packs the resulting six binary digits into a single byte index, and performs a linear search over a 26-element Grade-1 English Braille struct lookup table residing in dynamic RAM. The decoded character is displayed in real-time on the 16x2 I2C LCD and transmitted across the serial interface at 9600 baud.

Crucially, because the physical TCRT5000 breakout boards are wide (~10.2 mm x 5.8 mm), the six sensors are arranged on an 830-point solderless breadboard with an approximate 8 mm inter-sensor center-to-center pitch (rather than the standard 2.34–2.50 mm ANSI Braille cell spacing). As a result, the physical prototype was tested using a custom printed A4 paper sheet featuring oversized (6–8 mm) black circular dots spaced at an 8 mm pitch.

2.  WHAT IS ALREADY BUILT (PHYSICALLY VERIFIED)

Arduino Nano Microcontroller Core: ATmega328P MCU clocked at 16 MHz, 5V operating logic, CH340 USB-UART bridge, flashed using Arduino IDE 1.8.19 with 'ATmega328P (Old Bootloader)' setting on COM5.

Sensing Receptor Array: 6x TCRT5000 IR reflective sensor breakout modules mounted on an 830-point solderless breadboard. Digital Output (DO) pins are connected to Arduino GPIOs D2–D7. Onboard LM393 comparators calibrated manually via trim potentiometers.

I2C Alphanumeric Character Display: 16x2 HD44780-compatible LCD with PCF8574 I2C backpack (address 0x27) wired to hardware TWI pins A4 (SDA) and A5 (SCL), powered from the 5V and GND breadboard rails.

Embedded Firmware (Braille.ino): Complete Arduino sketch implementing non-blocking millis() debouncing (500 ms), 6-bit digitalRead() packing, boundary condition filtering (ignoring 0b000000 and 0b111111), and 2-row LCD cursor wrap-around logic.

Printed Test Medium: Custom A4 sheet with oversized 6–8 mm black circular dots arranged in 3x2 matrix format with an 8 mm pitch to match the breadboard sensor layout.

3.  WHAT IS NOT BUILT (UNIMPLEMENTED / PROPOSED ONLY)

Custom Manufactured PCB [NOT BUILT - DISCUSSED ONLY]: No custom PCB was fabricated via JLCPCB, EasyEDA, or KiCad. The prototype remains 100% wired on a breadboard.

Standard 2.5 mm ANSI Braille Cell Spacing [NOT BUILT]: The physical hardware does not have 2.5 mm sensor spacing; it has ~8 mm spacing due to breakout board dimensions.

DFPlayer Mini Audio Output Module [NOT BUILT - DISCUSSED ONLY]: No DFPlayer Mini board, MicroSD card, or 8 Ohm loudspeaker has been wired or tested.

Motorized Paper Feed Mechanism [NOT BUILT - DISCUSSED ONLY]: Paper transport is entirely manual; no stepper motor, servo, or drive rollers exist.

Analog Reflectance Acquisition (AO) [NOT BUILT]: The AO pins of all 6 TCRT5000 modules are completely unconnected. The system currently discards all analog signal intensity.

White-on-White Embossed Braille Recognition [NOT TESTED / NOT BUILT]: The system has only been tested on high-contrast black printed dots, not real monochromatic embossed cardstock.

4.  CURRENT HARDWARE ARCHITECTURE

| Component / Module | Module Pin | Arduino Nano Pin | Signal / Logic Function |
| --- | --- | --- | --- |
| Sensor 1 (Dot 1: Top Left) | DO (Digital Out) | Pin D2 | Digital Input (Black=HIGH, White=LOW) |
| Sensor 2 (Dot 2: Mid Left) | DO (Digital Out) | Pin D3 | Digital Input (Black=HIGH, White=LOW) |
| Sensor 3 (Dot 3: Bot Left) | DO (Digital Out) | Pin D4 | Digital Input (Black=HIGH, White=LOW) |
| Sensor 4 (Dot 4: Top Right) | DO (Digital Out) | Pin D5 | Digital Input (Black=HIGH, White=LOW) |
| Sensor 5 (Dot 5: Mid Right) | DO (Digital Out) | Pin D6 | Digital Input (Black=HIGH, White=LOW) |
| Sensor 6 (Dot 6: Bot Right) | DO (Digital Out) | Pin D7 | Digital Input (Black=HIGH, White=LOW) |
| Sensors 1 to 6 (Analog Pins) | AO (Analog Out) | UNCONNECTED | Unused continuous reflectance voltage |
| Sensors 1 to 6 (Power) | VCC / GND | 5V / GND Rails | Continuous 5V DC supply rail |
| 16x2 I2C LCD Backpack | SDA | Pin A4 | Hardware I2C Data (Address 0x27) |
| 16x2 I2C LCD Backpack | SCL | Pin A5 | Hardware I2C Clock (Address 0x27) |
| USB Mini Interface | USB Connector | USB Host (COM5) | 5V Power Supply + 9600 Baud Serial |

TABLE I: Actual Physical Prototype Wiring and Interface Mapping

5.  CURRENT SOFTWARE ARCHITECTURE

The existing firmware architecture relies on sequential polling of GPIO pins D2–D7, packing the 6 digital states into a single byte, and executing a linear search across an in-memory struct array. The exact firmware implementation is presented below:

#include <Wire.h>
#include <LiquidCrystal_I2C.h>

#define S1 2
#define S2 3
#define S3 4
#define S4 5
#define S5 6
#define S6 7

LiquidCrystal_I2C lcd(0x27, 16, 2);

struct BrailleMap {
  byte pattern;
  char character;
};

// 26-element Grade-1 Latin Alphabetic Table in SRAM
const BrailleMap table[] = {
  {0b100000,'A'}, {0b110000,'B'}, {0b100100,'C'}, {0b100110,'D'},
  {0b100010,'E'}, {0b110100,'F'}, {0b110110,'G'}, {0b110010,'H'},
  {0b010100,'I'}, {0b010110,'J'}, {0b101000,'K'}, {0b111000,'L'},
  {0b101100,'M'}, {0b101110,'N'}, {0b101010,'O'}, {0b111100,'P'},
  {0b111110,'Q'}, {0b111010,'R'}, {0b011100,'S'}, {0b011110,'T'},
  {0b101001,'U'}, {0b111001,'V'}, {0b010111,'W'}, {0b101101,'X'},
  {0b101111,'Y'}, {0b101011,'Z'}
};

int sizeTable = sizeof(table)/sizeof(table[0]);
byte lastPattern = 0;
unsigned long lastTime = 0;
int pos = 0;
const int debounce = 500;

void setup() {
  Serial.begin(9600);
  pinMode(S1, INPUT); pinMode(S2, INPUT); pinMode(S3, INPUT);
  pinMode(S4, INPUT); pinMode(S5, INPUT); pinMode(S6, INPUT);
  lcd.init(); lcd.backlight();
  lcd.setCursor(0,0); lcd.print("Braille Reader");
  delay(2000); lcd.clear();
}

byte readPattern() {
  byte p = 0;
  // POLARITY: BLACK DOT = HIGH (1), WHITE PAPER = LOW (0)
  if(digitalRead(S1)==HIGH) p |= 0b100000;
  if(digitalRead(S2)==HIGH) p |= 0b010000;
  if(digitalRead(S3)==HIGH) p |= 0b001000;
  if(digitalRead(S4)==HIGH) p |= 0b000100;
  if(digitalRead(S5)==HIGH) p |= 0b000010;
  if(digitalRead(S6)==HIGH) p |= 0b000001;
  return p;
}

char decode(byte p) {
  for(int i=0; i<sizeTable; i++) {
    if(table[i].pattern == p) return table[i].character;
  }
  return '?';
}

void display(char c) {
  if(pos >= 16 && pos < 32) {
    lcd.setCursor(pos-16, 1);
  } else if(pos >= 32) {
    lcd.clear(); pos = 0; lcd.setCursor(0, 0);
  } else {
    lcd.setCursor(pos, 0);
  }
  lcd.print(c);
  pos++;
}

void loop() {
  byte pattern = readPattern();
  if(pattern != lastPattern && millis() - lastTime > debounce) {
    if(pattern != 0 && pattern != 0b111111) {
      char ch = decode(pattern);
      if(ch != '?') {
        display(ch);
        Serial.print("Pattern: ");
        for(int i=5; i>=0; i--) Serial.print((pattern>>i)&1);
        Serial.print(" -> ");
        Serial.println(ch);
      }
    }
    lastPattern = pattern;
    lastTime = millis();
  }
  delay(100);
}

6.  CURRENT PROBLEMS & LIMITATIONS (EMPIRICALLY CONFIRMED)

Physical Grid Spacing Incompatibility: The TCRT5000 breakout board PCB width (~10.2 mm) physically prevents packing 6 modules into the standard 2.5 mm ANSI Braille cell layout on a breadboard. The current 8 mm spacing works only with custom printed sheets.

Inability to Read Standard Embossed Braille: Real Braille is monochromatic (white-on-white 0.48 mm physical dome relief). The onboard LM393 comparators cannot distinguish subtle diffuse relief scattering from flat cardstock.

Inter-Sensor Optical Crosstalk: Because all 6 IR LEDs are continuously illuminated in parallel, reducing the spacing between sensors causes wide-angle emitter light (+/-15 deg) to directly bleed into adjacent phototransistors.

Manual Threshold Brittleness & Drift: The six individual potentiometers require manual tuning. Thermal drift, ambient daylight variations, and minor standoff distance variations (+/-1 mm) cause false bit triggers.

Total Breakdown on Lateral Misalignment: Manual movement without alignment guides easily causes half-dot scanning. In a rigid Boolean lookup table, a single bit error results in an immediate '?' rejection or erroneous character substitution.

7.  AVAILABLE HARDWARE INVENTORY

| Hardware Component | Quantity | Physical Status / Availability |
| --- | --- | --- |
| Arduino Nano V3 (ATmega328P, CH340, Old Bootloader) | 1 | In hand, tested and functional |
| TCRT5000 Reflective IR Sensor Breakout Modules | 6 | In hand, DO pins functional, AO pins available |
| 16x2 HD44780 LCD + PCF8574 I2C Backpack | 1 | In hand, tested and functional @ address 0x27 |
| 830-Point Solderless Breadboard | 1 | In hand, currently houses prototype |
| Jumper Wires (Male-Male, Male-Female) | Assorted | In hand, functional |
| USB Mini Connection Cable | 1 | In hand, functional |
| DFPlayer Mini MP3 Module + Loudspeaker | Optional | [UNCERTAIN - VERIFY if purchased or discussed only] |

TABLE II: Inventory of Currently Available Hardware Components

8.  POTENTIAL HARDWARE DEVELOPMENT AREAS

H1. Analog Multi-Channel Signal Acquisition (AO Migration): Connect the existing TCRT5000 AO pins directly to the Arduino Nano ADC multiplexer channels (A0, A1, A2, A3, A6, A7), capturing the continuous 10-bit reflectance voltage rather than discarding it through LM393 1-bit comparators.

H2. Active Time-Multiplexed Emitter Modulation (TMOI): Rewire the IR LED anodes to digital pins D2–D7 to pulse each emitter individually, suppressing inter-sensor optical crosstalk in sub-3mm arrays without mechanical baffles.

H3. Micro-Aperture Spatial Collimator (MASC): Fabricate a 3D-printed or laser-cut aperture mask with 1.2 mm pinholes positioned over the sensor lenses to restrict the emission cone from +/-15 deg to +/-6.5 deg, matching the 1.44 mm ANSI Braille dot base diameter.

H4. Active Multi-Angle Cross-Scattering Matrix: By energizing emitter i while reading adjacent phototransistors j, the hardware captures the asymmetric 3D optical shadow cast across the 0.48 mm raised dome, enabling genuine white-on-white relief sensing.

9.  POTENTIAL SOFTWARE DEVELOPMENT AREAS

S1. Ambient-Subtracted Differential AC Filtering: Acquire phototransistor voltage with LED OFF (V_dark), pulse LED ON to acquire V_lit, and compute Delta V = V_lit - V_dark, achieving complete immunity to room lighting variations.

S2. Topographic Shadow Gradient Extraction Filter: Compute directional spatial gradients across the 6x6 cross-scatter matrix to decouple 0.48 mm physical dot elevation from surface albedo/color variations.

S3. Fixed-Point Soft-Decision Metric Decoder (SDMD): Evaluate continuous 6-dimensional analog vectors against ideal codeword centroids using an integer L1 Manhattan metric with nearest-codeword error recovery, replacing brittle Boolean table matching.

S4. Dynamic Substrate Self-Calibration Routine: Implement an automated baseline sampling routine that measures flat cardstock reflectance on startup and computes per-channel gain normalization factors.

10.  NOVELTY RESEARCH PLAN & SEARCH AREAS

To establish a genuinely defensible IEEE research paper, prior-art literature and patents must be systematically investigated across six specific focus areas:

1. Optical Crosstalk Suppression in Sub-3mm Discrete Arrays: Search IEEE Xplore, Optica, and patent databases for optical isolation and time-multiplexed emission in micro-scale phototransistor arrays.

2. Shape-from-Shadow Relief Extraction on 8-bit Microcontrollers: Investigate embedded implementations of photometric stereo and cross-scatter gradient deconvolution for surface relief profiling.

3. Non-Camera White-on-White Embossed Braille Readers: Survey academic literature for discrete optical transducers attempting to read unpainted embossed cardstock.

4. Soft-Decision Metric Classification in Constrained Embedded Systems: Examine integer Manhattan/Euclidean distance classifiers and nearest-neighbor error recovery on AVR/ARM microcontrollers.

5. Micro-Aperture Collimation for Reflective Optocouplers: Analyze beam-shaping techniques for GaAs IR LEDs (TCRT5000/ITR9909) using pinhole masks and 3D-printed baffles.

6. Patent Landscape (IPC G09B 21/00): Review patents concerning optical and optoelectronic reading aids for the visually impaired to ensure complete freedom to operate.

11.  INFORMATION THAT MUST BE VERIFIED BEFORE IMPLEMENTATION

1. Discrete Sensor Packages vs. Breakout Boards: Determine whether you will use bare 4-pin TCRT5000 components or modify the existing blue breakout boards to rewire the IR LED anodes for pulsed GPIO drive.

2. Fabrication Capabilities (3D Printing / Laser Cutting): Confirm access to 3D printing (SLA/FDM) or laser machining for fabricating the Micro-Aperture Collimator plate and paper alignment guide channel.

3. Real Embossed Braille Cardstock: Confirm access to standard Perkins-embossed or slate/stylus Braille cardstock (160 g/m^2) for true physical validation trials.

4. Audio Feedback Component Availability: Confirm whether the DFPlayer Mini MP3 board and speaker are physically available or if output should remain purely visual (16x2 I2C LCD + Serial UART).